SPEC NOTE DESCRIPTION: Covers IKO modified bituminous roofing for Adhesive Applied Roofing System - AARS

# General

## SECTION INCLUDES

### Materials and installation for SBS modified bituminous roofing system including roof curbs, pads, flashings and walkways for an Adhesive Applied Roofing System (AARS).

### System Description:

#### Substrate.

#### 2 ply Glass felt (mopped in asphalt).

#### Base polyisocyanurate insulation (mopped in asphalt).

#### Tapered polyisocyanurate insulation (mopped in asphalt).

#### Coverboard (mopped in asphalt).

#### 2 plies of SBS modified bitumen membrane (mopped in asphalt).

#### Flood coat and gravel surfacing.

### System Description:

#### Substrate.

#### 2 ply Glass felt (mopped in asphalt).

#### Base polyisocyanurate insulation (mopped in asphalt).

#### Tapered polyisocyanurate insulation (mopped in asphalt).

#### Coverboard (mopped in asphalt).

#### 2 plies of SBS modified bitumen membrane (base sheet mopped in asphalt, cap sheet heat welded).

### System Description:

#### Substrate.

#### Self-adhered vapour retarder.

#### Base polyisocyanurate insulation (adhered).

#### Tapered polyisocyanurate insulation (adhered).

#### Coverboard (adhered).

#### 2 plies of SBS modified bitumen membrane (self-adhered base, torch cap).

### System Description:

#### Substrate.

#### Self-adhered vapour retarder.

#### Base polyisocyanurate insulation (adhered).

#### Tapered polyisocyanurate insulation (adhered).

#### Coverboard (adhered).

#### 2 plies of SBS modified bitumen membrane (self-adhered).

### System Description:

#### Substrate.

#### Heat-welded vapour retarder.

#### Base polyisocyanurate insulation (adhered).

#### Tapered polyisocyanurate insulation (adhered).

#### Coverboard (adhered).

#### 2 plies of SBS modified bitumen membrane (heat welded).

### System Description:

#### Concrete Deck.

#### Heat-welded vapour retarder.

#### Base polyisocyanurate insulation (adhered).

#### Tapered polyisocyanurate insulation (adhered).

#### Coverboard (adhered).

#### 2 plies of SBS modified bitumen membrane (heat welded).

## RELATED REQUIREMENTS

SPEC NOTE: Edit the following paragraphs to list documents or Sections with specific information that the reader might expect to find in this Section, but is specified elsewhere. Do not include Division 00 or Division 01 Sections in this listing.

### Section 06 10 00 – Rough Carpentry

### Section 07 92 00 – Sheet Metal Flashing and Trim

## REFERENCE STANDARDS

SPEC NOTE: Edit the following paragraphs for this specific project. Confirm most recent dates of standards at Techstreet Store or other on-line market for reference standards.

### ASTM C1177/C1177M-17, Standard Specification for Glass Mat Gypsum Substrate for Use as Sheathing.

### CRCA Roofing Specifications Manual - current version at time of Building Permit.

### CSA A123.3-05 (R2015), Asphalt Saturated Organic Roofing Felts.

### CSA A123.21-20, Standard Test Method for the Dynamic Wind Uplift Resistance of Membrane-Roofing Systems

### CSA A123.23-15 (R2020), Product specification for polymer-modified bitumen sheet, prefabricated and reinforced.

### FM Approvals - Roofing Products.

### Material Safety Data Sheets (MSDS).

### CAN/ULC S107-10, Method of Fire Tests of Roof Coverings.

### CAN/ULC-S701-11, Standard for Thermal Insulation, Polystyrene, Boards and Pipe Covering.

### CAN/ULC-S702.1-2014-AMD1, Standard for Mineral Fibre Thermal Insulation for Buildings.

### CAN/ULC-S704-11, Standard for Thermal Insulation, Polyurethane and Polyisocyanurate Boards, Faced.

### CAN/ULC-S770-15- Standard Test Method for Determination of Long-term Thermal Resistance of Closed-Cell Thermal Insulating Foams.

## ADMINISTRATIVE REQUIREMENTS

### Convene pre-installation meeting four weeks prior to beginning waterproofing Work, with [DCC Representative] [roofing contractor's representative] [Departmental Representative] [Consultant] to:

#### Verify project requirements.

#### Review installation and substrate conditions.

#### Co-ordination with other building subtrades.

#### Review manufacturer's installation instructions and warranty requirements.

## SUBMITTALS

### Provide submittals in accordance with Section [01 33 00- Submittal Procedures].

SPEC NOTE: Only include the following paragraph where details of roofing are considered special, such as tapered insulation, flashing and control joints.

### Product Data:

SPEC NOTE: Include requests for relevant data to be furnished by the Contractor, before, during or after construction.

#### Provide most recent technical roofing components data sheets describing materials' physical properties and include product characteristics, performance criteria, physical size, finish and limitations.

#### Provide WHMIS MSDS and indicate VOC content for:

##### Primers.

##### Sealers.

### Provide shop drawings:

#### Indicate adheseive pattern for Field, Perimeter Zone and Corner Zone.

#### Indicate [control joints,] [flashing,] [tapered insulation] details.

SPEC NOTE: Use the following paragraph when tapered insulation is specified. Expand requirements for shop drawings to suit project.

#### Provide layout for tapered insulation.

SPEC NOTE: Use the following paragraph when installing PMR assembly and when samples are required for selection of finish, colour, texture, and other physical requirements.

### Manufacturer's Certificate: certify that [products] meet or exceed [specified requirements] .

### Test and Evaluation Reports: submit laboratory test reports certifying compliance of [bitumens] [membrane] [roofing felts] with specification requirements.

### Manufacturer's Installation Instructions: indicate special precautions required for seaming membrane.

SPEC NOTE: Use the following two paragraphs when a manufacturer's warranted roof system is specified.

### Manufacturer's field report: in accordance with Section 01 45 00- Quality Control.

### Installer’s proof of manufacturer’s required training program and registration.

### Reports: indicate procedures followed, ambient temperatures and wind velocity during application.

### Warranty: submit sample warranty documents meeting specified warranty requirements.

### Sustainability Submittals:

#### EPD’s in accordance with ISO 14025 and rely on Life Cycle Assessment (LCA) as follows:

##### APP-Modified Bitumen Roofing Membrane.

##### SBS-Modified Bitumen Roofing Membrane: Cold Adhesive

##### SBS-Modified Bitumen Roofing Membrane: Hot Asphalt

##### SBS-Modified Bitumen Roofing Membrane: Torch Applied

##### SBS-Modified Bitumen Roofing Membrane: Self Adhered

## QUALITY ASSURANCE

### Installer qualifications: company or person specializing in application of modified bituminous roofing systems with minimum five (5) years documented experience.

SPEC NOTE: When either the Diamond Shield or Modified Bitumen/BUR Limited Labour and Membrane Warranty is selected use the following:

### Installer qualifications: IKO MODBIT/BUR Registered Applicator (IAAP).

### Qualification of Workers:  Employ skilled applicators certified by Provincial Apprenticeship Board as being ‘enrolled in’ or ‘having completed’ appropriate training of installation of modified bituminous membrane roofing.

### Roofing inspections:

#### 3rd party inspections; paid for by [Work of this section [Owner] to satisfy conditions of 3rd Party Guaranties.

## FIRE PROTECTION

### Fire Extinguishers:

#### Maintain one stored pressure rechargeable type with hose and shut-off nozzle,

#### ULC labelled for A, B and C class protection.

#### Size[s] [1.14] [4.5] [9] [14] [2.25] kg [or as indicated] on roof per torch applicator, within 6 m of torch applicator.

### Maintain fire watch for (1) hour after each day's roofing operations cease.

## DELIVERY, STORAGE, AND HANDLING

### Deliver, store and handle materials in accordance with manufacturer's written instructions.

### Storage and Handling Requirements:

#### Safety: comply with requirements of Workplace Hazardous Materials Information System (WHMIS) regarding use, handling, storage, and disposal of asphalt, sealing compounds, primers and caulking materials.

#### Provide and maintain dry, off-ground weatherproof storage.

#### Store roll material in upright position. Store membrane rolls with salvage edge up.

#### Remove only in quantities required for same day use.

#### Place plywood runways over completed Work to enable movement of material and other traffic.

#### Store sealants at +5 degrees C minimum.

#### Store insulation protected from weather and deleterious materials.

### Waste Management: in accordance with Section 01 74 19- Waste Management and Disposal. Return surplus materials to manufacturer whenever possible

## SITE CONDITIONS

### Ambient Conditions:

#### In strict compliance with manufacturer’s requirements.

### Install roofing on dry deck, free of snow and ice, use only dry materials and apply only during weather that will not introduce moisture into roofing system.

## WARRANTY

SPEC NOTE: Select either Diamond Shield or Limited Labour and Membrane warranty

### IKO Diamond Shield [10] [15] [20] Year Warranty.

#### Warranty to include coverage for labour and materials required to complete repair or replacement of material found to have a manufacturing defect or workmanship to install membrane resulting in roof leaks

#### Provide cost of premium for warranty.

### IKO Limited Labour and Membrane [10] [15] [20] Year Warranty.

#### Warranty to include coverage for labour and materials required to complete repair or replacement of material found to have a manufacturing defect resulting in roof leaks.

# Products

## MANUFACTURERS

### Acceptable Manufacturers of system materials:

#### IKO Industries.

## PERFORMANCE CRITERIA

SPEC NOTE: the CSA A123.21 test for wind uplift resistance is available from the NRC website. Search Wind-RCI Calculator.

### Roofing System: adhesive pattern to CSA A123.21 for wind uplift resistance as follows:

#### Field: [_____].

#### Perimeter Zone: [_____].

#### Corner Zone: [_____].

### Fire performance to CAN/ULC S107: Class [A] [B][ [C].

## DECK OVERLAY BOARD

### Glass Mat, Gypsum Board: to ASTM C1177, thickness as indicated on drawings.

## VAPOUR RETARDER AND PRIMER

SPEC NOTE:

### Self-adhesive applied vapour retarder: non-woven glass fiber mat permeated with SBS modified bitumen.

#### Acceptable Materials:

##### MVP Sand.

### Heat welded vapour retarder:

#### Acceptable Materials:

##### Torchflex TF-95-SF-Base; to CSA A123.23-15, Type A, Grade 3.

### Primer/adhesive: self-adhering membrane adhesive.

#### Acceptable Materials:

##### S.A.M. Adhesive.

## MEMBRANE MATERIALS

SPEC NOTE: Select the following membranes when fire resistance is required.

### Cap Sheet and cap sheet flashing: heat welded.

#### ULC Fire Resistance: Class A.

#### Properties: to CSA A123.23-15, Type B or C Grade 1.

#### Colour: to be selected by Consultant from manufacturer’s standard range.

#### Acceptable Materials:

##### Prevent TP-250 Cap.

##### Prevent TP-HD Cap.

### Cap Sheet and cap sheet flashing: heat welded.

#### ULC Fire Resistance: Class C.

#### Colour: to be selected by Consultant from manufacturer’s standard range.

SPEC NOTE: Select from the four cap sheet options below when fire resistance in not required.

#### Acceptable Materials:

##### Torchflex TP-180-Cap.

###### Properties: to CSA A123.23-15 Type B Grade 1, non-woven polyester reinforcement mat strengthened with select glass fibre strands.

##### Torchflex TP-250 Cap.

###### Properties: to CSA A123.23-15 Type B Grade 1, non-woven polyester reinforcement mat strengthened with select glass fibre strands.

###### Thickness: 4 mm.

##### Torchflex TP-250 Cap (5.0).

###### Properties: to CSA A123.23-15 Type B Grade 1, non-woven polyester reinforcement mat strengthened with select glass fibre strands.

###### Thickness: 5 mm.

##### Torchflex TP-HD-Cap.

###### Properties: to CSA A123.23-15 Type C Grade 1, non-woven polyester reinforcement with glass fibre scrim in both machine and cross directions.

###### Thickness: 4 mm.

SPEC NOTE  ArmourCool has reflective mineral granules to 82 SRI index which exceeds LEED requirement of 78,

##### ArmourCool Granular TP-HD.

###### Properties: to CSA A123.23-15 Type C Grade 1, non-woven polyester reinforcement with glass fibre scrim in both machine and cross directions.

###### SRI index: 82.

###### Thickness: 5 mm.

SPEC NOTE: Select the following when self-adhering cap sheet membrane is required. ArmourStick Cap sheet must be used with ArmourStick HD Base.

### Self-adhering Cap Sheet and Cap Sheet flashing:

#### Acceptable Materials:

##### ArmourStick HD-Cap peel and stick cap sheet.

###### Properties: to CSA A123.23 Type C Grade 1 and ASTM 6162 Type I, Grade G.

###### Thickness: 4.5 mm.

SPEC NOTE: Select ArmourStick HD Base sheet for use with ArmourStick Cap sheet.

### Self-Adhered Base sheet:

#### ArmourStick HD-Base peel and stick base sheet; to CSA A123.23 Type C, Grade 3.

#### Amourbond 180 self-adhering base sheet; to CSA A123.23 Type B, Grade 3.

#### Armourvent Base self-adhering base sheet; to CSA A123.23 Type A, Grade 3.

### Heat Welded Base Sheet:

#### Torchflex TP-180-FF-Base: to CSA A123.23 Type B, Grade 3.

#### Torchflex HD-FF-Base, to CSA A123.23 Type C, Grade 3.

### Asphalt Adhered Base Sheet:

#### Modiflex MP-180-FS-Base; to CSA A123.23 Type B, Grade 3.

#### Modiflex MP-180-SS-Base; to CSA A123.23 Type B, Grade 3.

### Self-Adhering Base Sheet Flashing:

#### Properties: non-woven reinforced polyester mat strengthened with select glass fibre strands.

#### Thickness: 2.5 mm.

#### Acceptable Materials:

##### Armourbond Flash.

SPEC NOTE: Felt can be served as a vapour retarder or as a ply sheet forming multi-ply built up roofing system.

## GLASS FELT

### Asphalt saturated Glass mat to CSA A123.17 and ASTM D2178 Type 6.

#### Acceptable Materials:

##### IKO Type 6 Glass Ply Sheet

## COMPOSITE OVERLAY BOARD

### Composite Cover Board: factory laminated base sheet membrane and cover board.

#### Acceptable Materials:

##### Protectobase 180.

## INSULATION OVERLAY BOARD

### Adhered board with mineral-fortified asphaltic core and high-strength reinforcing glass fibre mat facers.

#### Thickness: 4.8 mm (3/16”).

#### Acceptable Materials:

##### Protectoboard.

### Rigid high compressive strength polyisocyanurate foam insulation with glass fiber facers:

#### Acceptable Materials:

##### IKOTherm CoverShield.

## LAMINATED COVER BOARD

### High density polyisocyanurate insulation factory laminated to 180 weight polyester reinforced SBS modified bitumen base sheet; with dual selvage self-adhering edges.

#### Acceptable Materials:

##### SheildBase 180.

## POLYISOCYANURATE INSULATION

SPEC NOTE:

### To CAN/ULC-S704, Type 2 and ASTM C1289 Type II Class 2, Grade 2.

### Long Term Thermal Resistance (LTTR): to CAN/ULC S770.

### Equal Thickness material:

#### Thickness as indicated on drawings.

#### Acceptable Materials:

##### IKOTherm III.

### Tapered:

#### Acceptable Materials:

##### IKOTherm III Tapered.

## ADHESIVE

### Roof board and insulation adhesive: highly elastomeric, low rise foamable, solvent-free.

#### Acceptable Materials:

##### IKO Millennium Adhesive.

### Field adhesive; solvent-free, single component cold-applied interply adhesive for use on field application of membrane.

#### Acceptable Materials:

##### Cold Gold Field Adhesive.

### Flashing adhesive for steep slopes; solvent-free,single component, asphalt modified urethane adhesive for flashing application of membrane.

#### Acceptable Materials:

##### Cold Gold Flashing Adhesive.

## ASPHALT

### Liquid or keg asphalt material to CSA A123.40-M, Type III.

#### Acceptable Materials:

##### Asphalt Type 3 – oxidized bitumen.

## ACCESSORY MATERIALS

### Self-adhering membrane adhesive: quick-drying, solvent-based surface preparation.

#### S.A.M. Adhesive.

### Asphalt roofing primer:

#### IKO Standard Asphalt Primer.

### Mastic: modified asphalt sealant to ASTM D4586.

#### AquaBarrier Mastic.

### Mod-Bit Primer: solvent base primer.

#### IKO Mod-Bit Primer.

### Roofing Tapes;

#### Fire resistant: inorganic reinforcing mat of high strength non-woven glass fibres.

##### Modiflex Roof Tapes.

#### SBS polymer glass fibre reinforced roofing tape:

##### TorchTape 180 FF.

### Liquid Waterproofing: solvent free, liquid applied membrane coating.

#### MS Detail.

## WALKWAYS

SPEC NOTE: Walkways on roofs are intended to protect roof from damage and provide directed access to equipment etc.

### Walkways to consist of one additional ply of cap sheet membrane. Colour to be different from field membrane as selected by [Departmental Representative] [DCC Representative] [Consultant].

## CARPENTRY

### Materials: refer to Section 06 10 00 Rough Carpentry.

# Execution

SPEC NOTE:

## QUALITY OF WORK

### Examine and prepare roofing Work in accordance with [CRCA Roofing Specification Manual] [Roofing Manufacturer's Specification Manual] [[____] [Provincial]<options>Territorial</options >Roofing Association Manual] , particularly for fire safety precautions, and to [FM] [ULC] Design No. [_____]

### Assembly, component and material connections will be made in consideration of appropriate design loads, with reversible mechanical attachments.

## EXAMINATION OF ROOF DECKS

### Verification of Conditions:

#### Review with [Consultant] [DCC Representative] [Departmental Representative] deck conditions including parapets, construction joints, roof drains, plumbing vents and ventilation outlets to determine readiness to proceed.

### Evaluation and Assessment:

#### Prior to beginning of work ensure:

##### Decks are firm, straight, smooth, dry, free of snow, ice or frost, and swept clean of dust and debris. Do not use calcium or salt for ice or snow removal.

##### Curbs have been built.

##### Roof drains have been installed at proper elevations relative to finished roof surface.

##### Plywood and lumber nailer plates have been installed to deck, walls and parapets as indicated.

### Do not install roofing materials during rain or snowfall.

## PROTECTION OF IN-PLACE CONDITIONS

### Cover walls, walks [, slopped roofs] and adjacent work where materials hoisted or used.

### Use warning signs and barriers. Maintain in good order until completion of Work.

### Clean off drips and smears of bituminous material immediately.

### Dispose of rain water off roof and away from face of building until roof drains or hoppers installed and connected.

### Protect roof from traffic and damage. Comply with precautions deemed necessary by [Departmental Representative] [DCC Representative] [Consultant].

### At end of each day's work or when stoppage occurs due to inclement weather, provide protection for completed Work and materials out of storage.

### Metal connectors and decking will be treated with rust proofing or galvanization.

SPEC NOTE: not required by IKO. Use the following only where acoustic deck is specified.

## PREPARATION OF STEEL DECK (CHANNEL TYPE)

### Install sound absorbing insulation in flutes of acoustical steel roof deck in accordance with deck manufacturer's instructions.

## DECK OVERLAY BOARD

### Mechanically fasten or adhere deck overlay in accordance with CSA A123.21 tested assembly to deck to pre-secure material prior to installation of vapour retarder.

### Place with long axis of each sheet transverse to steel deck ribs, with end joints staggered and fully supported on ribs.

## PRIMING DECK

### Apply deck primer to roofing substrate at the rate recommended by manufacturer.

## VAPOUR RETARDER

### Lay material and secure to substrate in accordance with manufacturer’s requirements.

## INSULATION APPLICATION

### Tapered insulation application:

#### Install tapered insulation as [second] [first] insulation layer, in accordance with shop drawings. Stagger joints between layers 150 mm minimum.

### Insulation: adhered application:

#### Install in multiple layers of no more than [______] mm per layer.

#### Joint: staggered minimum 300 mm

#### Apply adhesive in accordance with manufacturer’s written recommendations.

#### Place boards in parallel rows with ends staggered, and in firm contact with one another.

#### Cut end boards to suit.

### Hot asphalt application:

#### Install in multiple layers of no more than [______] mm per layer.

#### Joint: staggered minimum 300 mm.

#### Place boards in parallel rows with ends staggered, and in firm contact with one another.

#### Cut end boards to suit.

#### Use hot asphalt between layers as recommended by manufacturer.

## OVERLAY BOARD APPLICATION

### Apply overlay board over insulation and pre-secure in accordance with manufacturer’s written instructions. Apply with adhesive or hot asphalt depending on the type of system being applied.

SPEC NOTE: When composite overlay board material is used delete base sheet application.

## COMPOSITE OVERLAY BOARD APPLICATION

### Install materials in accordance with manufacturer’s written instructions.

### Apply with adhesive or hot asphalt depending on the type of system being applied; to CSA A123.21 for wind uplift resistance and in accordance with reviewed Shop Drawings.

## BASE SHEET APPLICATION

### Install materials in accordance with manufacturer’s written instructions.

### Starting at low point of roof, perpendicular to slope, unroll base sheet and heat weld.

## CAP SHEET APPLICATION

### Install materials in accordance with manufacturer’s written instructions for heat welding.

### Starting at low point on roof, perpendicular to slope, unroll cap sheet and heat weld.

### Application to be free of blisters, fishmouths and wrinkles.

## FLASHINGS

### Complete installation of flashing base sheet stripping prior to installing membrane cap sheet.

SPEC NOTE: select one of the appropriate methods below in square brackets. Use hot air welding for self-adhered system.

### Lap flashing base sheet to membrane base sheet minimum 200 mm and seal by [mopping] [torch welding] [hot air welding].

SPEC NOTE: select one of the appropriate methods below in square brackets. Use hot air welding for self-adhered system.

### Lap flashing cap sheet to membrane cap sheet 150 mm minimum and [torch weld] [hot weld].

### Provide 75 mm minimum side lap and seal.

### Properly secure flashings to their support, without sags, blisters, fishmouths or wrinkles.

### Roof penetrations:

#### Install roof drain pans, vent stack covers and other roof penetration flashings and seal to membrane in accordance with manufacturer's recommendations and details.

## WALKWAYS

### Install walkway in accordance with manufacturer's instructions and as indicated.

### Apply primer to cap sheet membrane and torch apply, ensuring selvage edge is removed.

## FIELD QUALITY CONTROL

### Inspections:

SPEC NOTE: Use the following paragraphs  for Federal Government projects when roofing inspection service to be employed.

#### Inspection and testing of roofing application will be carried out by testing laboratory designated by [Consultant] [Departmental Representative] [DCC Representative] .

#### [DCC Representative] [Consultant] [Departmental Representative] will pay for tests as specified in Section [01 45 00- Quality Control] .

SPEC NOTE: Use the following paragraphs for Private Sector projects.

SPEC NOTE: Inspection agency must be fully experienced with membrane and installation procedures.

#### Inspection and testing of roofing application will be carried out by testing laboratory designated by [Consultant] [DCC Representative] [Departmental Representative] .

#### Costs of tests will be paid [under cash allowance] [by Owner] .

### Testing:

SPEC NOTE: Various types of testing may be required to ensure that completed roofing system complies with specified requirements. Specify testing methods, and expected results here.

#### [_____]

## CLEANING

### Remove bituminous markings from finished surfaces.

### In areas where finished surfaces are soiled caused by work of this section, consult manufacturer of surfaces for cleaning advice and complying with their [documented] instructions.

### Repair or replace defaced or disfigured finishes caused by work of this section.

### Waste Management: separate waste materials into three streams – reuse, recycle, waste - in accordance with Section 01 74 19- Waste Management and Disposal.

END OF SECTION